| Vessel: Bitu Atlantic |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Date (dd/mm/yy): 31/12/18 |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| List Serial No: Year/xxx: 2018 / 006 |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| Date defect | Defect No. D= deck E= Engine D or E year/ xxx | Priority yes / no | Class Item yes/no | open / closed/ in progress | Description / Details of the defect |  | Estimated date of completion |  | REQ number | REQ delivered Yes / no |  | Ships' Comments / Date completed |  | Work to be carried out by -ship staff -riding gang -shore assistance |  | REMARKS by Superintendent or upon completion CORRECTIVE ACTION taken |  |  |
| 15/04/18 | E |  | Y | Closed | M/E Due for O/H. As per R.Hours |  | 30.09.18 |  | ATL/022E | Yes |  |  |  | ship staff |  | Done by Ship staff |  |  |
| 15/04/18 | E |  | Y | Closed | A/E Due for OverH. As per R.Hours |  | 30.09.18 |  | ATL/006E ATL/007E ATL/008E ATL/009E | Yes |  |  |  | ship staff |  | Done by Ship staff |  |  |
| 15/04/18 | E |  | Y | Closed | All 3 Aux. Alternators need to be O/H |  | 06.07.18 |  | 10.07.18 | Yes |  |  |  | ship staff |  | Alternator no-1 and 2 -3 completed cleaning with chemical and air gap clearance taken and recorded and inspection |  |  |
| 15/04/18 | E |  | Y | Closed | Em’cy. Alternator to be O/H |  | 15.08.18 |  |  |  |  |  |  | ship staff |  | Done by Ship staff |  |  |
| 15/04/18 | E |  |  | Closed | All Suc. & O/B V/V need to be O/H |  | 30.09.18 |  | N/A |  |  |  |  | shore assistance |  | Done by Shipyard |  |  |
| 15/04/18 | E |  |  | Closed | Water wash of Furnaces and tubes |  | 15.07.18 |  | ATL/017E | Yes |  | 25.05.2018 |  | ship staff |  | Done by Ship staff |  |  |
| 15/04/18 | E |  |  | Closed | In Steam Line all V/V to be O/H. |  |  |  |  |  |  | 20.10.2018 |  | ship staff |  | Done by Ship staff |  |  |
| 15/04/18 | E |  |  | Closed | All steam line defective laggings to be rectify |  |  |  | ATL/015E | Yes |  | 20.10.2018 |  | ship staff |  | Done by Ship staff |  |  |
| 15/04/18 | E |  |  | Closed | M/E exhaust gas pipe lines bellow to be change. |  |  |  | N/A |  |  | 22.05.2018 |  | Shore assistance |  | Done by Shipyard staff |  |  |
| 15/04/18 | E |  |  | Closed | Drain cooler tubes to be clean and anodes to be renewed. |  |  |  | N/A |  |  | 30.05.2018 |  | Shore assistance |  | Done by Shipyard staff |  |  |
| 15/04/18 | E |  |  | Closed | M/E Jacket cooler No – 1 & 2 to be clean. |  | 30.09.18 |  | N/A |  |  | 20.10.2018 |  | Ship staff |  | Done by Shipyard staff |  |  |
| 15/04/18 | E |  |  | Closed | Acc. Air Cond. Compressor clean condenser and anodes renewal |  |  |  | N/A |  |  | 27.07.2018 |  | Ship staff |  | Done by Ship staff |  |  |
| 15/04/18 | E |  |  | Closed | Acc. Air Cond. Compressor –change evaporator |  |  |  | ATL/016E | Yes |  | 06.04.2018 |  | Shore assistance |  | Done by Shipyard staff |  |  |
| 15/04/18 | E |  |  | Closed | Refrig. No-1&2 Compressor clean condenser and anodes renewal |  |  |  | N/A |  |  | 06.06.2018 |  | Ship Staff |  | Done by Ship staff |  |  |
| 15/04/18 | E |  |  | Closed | No. 2 Main Air Reservoir Inspection |  |  |  | N/A |  |  | 07.05.2018 |  | Ship Staff |  | Done by Ship staff |  |  |
| 15/04/18 | E |  |  | Closed | Aux. Boiler Safety Valve O/H, Tested and Calibrated |  |  |  | N/A |  |  | 10.05.2018 |  | Shore assistance |  | Done by Shipyard staff |  |  |
| 15/04/18 | E |  |  | Closed | Econom. Boiler Safety Valve O/H, Tested and Calibrated |  |  |  | N/A |  |  | 10.05.2018 |  | Shore assistance |  | Done by Shipyard staff |  |  |
| 15/04/18 | E |  |  | Closed | Cascade Tank - cleaning |  |  |  | N/A |  |  | 10.05.2018 |  | Ship Staff |  | Done by Ship staff |  |  |
| 15/04/18 | E |  |  | Closed | MGPS Valves to be O/H |  |  |  | N/A |  |  | 01.05.2018 |  | Ship Staff |  | Done by Ship staff |  |  |
| 15/04/18 | E |  |  | Closed | ME FO Q.C.Valve Inlet – O/H |  |  |  |  |  |  | 04.07.2018 |  | Ship Staff |  | Done by ship staff |  |  |
| 15/04/18 | E |  |  | Closed | C.W Expansion Tank cleaning |  |  |  | N/A |  |  | 10.05.2018 |  | Ship Staff |  | Done Ship staff |  |  |
| 15/04/18 | E |  |  | Closed | Service and Settling FO tanks Cleaning |  |  |  | N/A |  |  |  |  | Shore assistance |  | Done by ship staff |  |  |
| 15/04/18 | E |  |  | Closed | No1. P and No. 1 Stbd . FO Stor. Tanks - cleaning |  |  |  | N/A |  |  |  |  | Shore assistance |  | Done by Shipyard staff |  |  |
| 15/04/18 | E |  |  | Closed | ME L.O Cooler - cleaning |  |  |  | N/A |  |  | 30.06.2018 |  | Ship Staff |  | Done by Ship staff |  |  |
| 15/04/18 | E |  |  | Closed | ME FO filters – cleaning |  |  |  | N/A |  |  | 31.05.2018 |  | Ship Staff |  | Done by Ship staff |  |  |
| 15/04/18 | E |  |  | Closed | ME LO filters - cleaning |  |  |  | N/A |  |  | 30.06.2018 |  | Ship Staff |  | Done by Ship staff |  |  |
| 15/04/18 | E |  |  | Closed | FWG - cleaning |  |  |  | N/A |  |  | 30.04.2018 |  | Ship Staff |  | Done by Ship staff |  |  |
| 15/04/18 | E |  |  | Closed | Sea chests cleaning Filters high /low suction |  |  |  | N/A |  |  | 04.06.2018 |  | Ship Staff |  | Done by Ship staff |  |  |
| 15/04/18 | E |  |  | Closed | L.O cooler for AE 1,2 & 3 to be clean. |  |  |  | N/A |  |  | 23.07.2018 |  | Ship Staff |  | Done by Ship staff |  |  |
| 15/04/18 | E |  |  | Closed | Air cooler (F/w & Air side) for AE 1,2 & 3 to be clean. |  |  |  | N/A |  |  | 22.07.2018 |  | Ship Staff |  | Done by Ship staff |  |  |
| 15/04/18 | E |  |  | Closed | Main Air Compressor 1 &2 need to be O/H. |  | 10.07.18 |  | ATL/014E | Yes |  |  |  | Ship Staff |  | Done by Ship staff |  |  |
| 15/04/18 | E |  |  | Closed | Steam Boiler tube side to be pressure test. |  |  |  | N/A |  |  | 02.07.2018 |  | Ship Staff |  | Done by ship staff |  |  |
| 15/04/18 | E |  |  | Cancel | Navigation lights to be renewed |  |  |  |  |  |  | May, 2018 |  | Ship Staff |  | Done by Ship staff |  |  |
| 15/04/18 | E |  |  | Closed | Pneumatic V/V of Em’cy Fire P/P Suction side to be O/H. |  | 02.07.18 |  |  |  |  | 12.07.2018 |  | Ship Staff |  | Done by ship staff |  |  |
| 15/04/18 | E |  |  | Closed | Pressure and Temp. Gauge Calibration + Certificate |  |  |  | N/A |  |  | 20.10.2018 |  | Ship Staff |  | Done by ship staff |  |  |
| 15/04/18 | E |  |  | Closed | Engine Crane –Test + Certificate |  |  |  | N/A |  |  | 20.10.2018 |  | Shore assistance |  | Done by ship staff |  |  |
| 15/04/18 | E |  |  | Closed | Slings and Lifting Device – Test + Certificate |  |  |  | N/A |  |  | 20.10.2018 |  | Shore assistance |  | Done by ship staff |  |  |
| 15/04/18 | E |  |  | Closed | Air Condition Refrig. S.W. Pump - Overhauling |  |  |  | ATL/018E | Yes |  | 25.06.2018 |  | Ship Staff |  | Done by ship staff |  |  |
| 15/04/18 | E |  |  | Closed | Fire & General Service Pump - Overhauling |  |  |  | ATL/019E | Yes |  | 20.05.2018 |  | Ship Staff |  | Done by Shipyard staff |  |  |
| 15/04/18 | E |  |  | Closed | Cooling S.W. Pump |  |  |  | ATL/020E | Yes |  | 14.05.2018 |  | Ship Staff |  | Done by Shipyard staff |  |  |
| 15/04/18 | E |  |  | Closed | No. 2 F.O. Circulation Pump – Overhauling |  |  |  | ATL/021E | Yes |  | 20.10.2018 |  | Ship Staff |  | Done by Shipyard staff |  |  |
| 15/04/18 | E |  |  | Closed | Various Pipes in E/R to be replace and eliminate lacking |  |  |  | N/A |  |  | 16.05.2018 |  | Ship Staff |  | Done by Shipyard staff |  |  |
| 15/04/18 | E |  |  | Closed | All purifiers to be clean and OH according RH schedule. |  |  |  | ATL/010E ATL/011E ATL/012E | Yes |  | 20.10.2018 |  | Ship Staff |  | Done by ship staff |  |  |
| 01/11/18 | E | Yes |  | Open | Main engine fuel oil viscosity control unit – steam control valve out of order |  | 15.12.18 |  | ATL/162E |  |  |  |  | Ship Staff |  |  |  |  |
| 04/11/18 | E | Yes |  | Open | Steam boiler feed water pump no. 2 electromotor burn |  | 15.12.18 |  |  |  |  |  |  | Shore assistance |  |  |  |  |
| 12/11/18 | E | Yes |  | Open | MGPS chlorine attacker out of order |  | 15.12.18 |  | ATL/165E |  |  |  |  | Ship Staff |  |  |  |  |
| 09/12/18 | E | Yes |  | Closed | FWG Evaporator tubes partially blocked |  | 25.12.18 |  |  |  |  | 28.12.18 |  | Ship Staff |  |  |  |  |
| 19/12/18 | E | Yes |  | Closed | Steam pipe flange gasket for boiler FO heater before heater at ECR platform stbd side badly leaking. |  | 22.12.18 |  |  |  |  |  |  | Ship Staff |  |  |  |  |
| 31/12/18 | E | Yes |  | Open | Steering Gear Room sound power telephone not working due to defective telephone PCB. |  |  |  | Req’n to be raised shortly |  |  |  |  | Ship Staff |  |  |  |  |