Vessel’s Name:   Bitu Atlantic                                                            Date (dd/mm/yy) :  31/12/18

| Component | Preventive Maintenance | Maintenance Interval | Last Record |
| --- | --- | --- | --- |

| Main Engine main Components (Piston, Liner, Heads) | Overhaul Engine | According to running hours and makers instructions | 23.10.2018 |
| --- | --- | --- | --- |
| Fuel Valves | Pressure test, exchange nozzles | According to engine performance approx. Every 6000 hrs | 23.10.2018 |
| Turbo Charger | General Overhaul, compressor inner components | According to running hours and in line with engine overhaul | 23.10.2018 |
| Engine Safety Equipment | Overhaul & Setting | Every engine overhaul | 23.10.2018 |
| Stern Tube System | Check wear down clearance | Every five years | 03.10.2018 |
| Aux. Engines | General overhaul, top overhaul | According to running hours and makers instructions | 23.10.2018 |
| Pumps | Check slip ring Clearance, bearing, Housing, coupling | Classification Continuous survey | 23.10.2018 23.10.2018 |
| Electrical Equipment Switchboard | Visual inspection for damaged cables, contacts, breakers, penetrations | Every 12 months and dry-docking | 27.12.2018 |
| Alternators | Visual inspection, Test reverse power, Change bearings Insulation reading cleaning | Classification | 28.12.2018 |
| Bilge System | Visual inspection, Clean filter | Every month | 16.12.2018 |
| Steering System | Visual Inspection for leaks, Note pump pressure, Measure rudder slap | Whilst running Whilst running Every 6 months | 28.12.2018 23.10.2018 |
| Electrical Motors & Starters | Insulation readings, Change bearings, Inspect contacts & terminals | Annually | 30.12.2018 |
| Air System | Check Safety valve Setting, overhaul valves, check bearings | Classification Continuous survey, every dry docking | 23.10.2018 23.10.2018 |
| Dock / Mooring Equipment | Visual inspection | Every month | 29.12.2018 |
| Bilge / Ballast System | Visual inspection pipes, valves | Every month | 27.12.2018 |
| Fuel system | Visual inspection Change filters | Every month | 28.12.2018 |
| Oily water separator | Drain Separator & Flush, Check 3-way valves Check 15 ppm alarm | Every 6 months | 23.10.2018 29.12.2018 29.12.2018 |
| Navigation Equipment | Visual inspection Overhaul | Every month | 23.12.2018 |
| Measurement Equipment |  |  | 23.10.2018 |

| P. MAITI |  |  |
| --- | --- | --- |
| Name Chief Engineer |  | Signature |