| Vessel: Bitu Atlantic |  | Date (dd/mm/yy): 31/01/19 |
| --- | --- | --- |

|  | Date | HFO (mt) / Sulphur (%) | MDO (mt) / Sulphur (%) |
| --- | --- | --- | --- |
| # Previous Month End ROB | 31.12.2018 | 652.0 / 2.10 | 94.6 / 0.0452 |
|  | 29.01.2019 | 750.0 / 3.34 | 290.0 / 0.085 |
| Bunkered | NA |  |  |
|  | NA |  |  |
| Total Bunkered this Month |  | 750.0 | 290.0 |
| Total Bunkers ROB | 31.01.2019 | 818.0 | 314.0 |
| Consumption this Month |  | 584.0 | 70.6 |

| Total Main Engine HFO Cons this Month | 443.6 |
| --- | --- |
| Total Main Engine DO Cons this Month | 4.13 |
| Total Aux Engines HFO Cons this Month | 59.4 |
| Total Aux Engines DO Cons this Month | 29.91 |
| Total Thermal & Aux Boilers HFO Cons this Month | 81.0 |
| Total Thermal & Aux Boilers DO Cons this Month | 36.56 |

| ## Tank Soundings |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- |
| HFO |  |  | MDO |  |  |
| Tank (nr.) | U/Sounding (cm) | Content (m³) / % Sulphur Content | Tank (nr.) | Sounding (cm) | Content (m³) / % Sulphur Content |
| HFO #1 Port side | 236.6 | 378.23/ 3.34 | DO Port side | 0.00 | 0.00 |
| HFO #2 Port side | 220.1 | 398.66/ 3.34 | DO #1 Stbd side | 1297 | 336.6/ 0.085 |
| HFO #2 Stbd side | 1217 | 10.05 / 2.89 | DO Service | Gauge | 6.50 / 0.0452 |
| HFO Settling | Gauge | 14.5 / 2.89 | LSMGO Service | Gauge | 6.60 / 0.0452 |
| HFO Service | Gauge | 17.0/ 2.89 | DO TOB Service | Gauge | 10.1 / 0.0452 |
| HFO Boiler service | Gauge | 7.5 / 2.89 |  |  |  |
| HFO TOB Service | Gauge | 18.5/ 2.89 |  |  |  |
| HFO Overflow | 388.1 | 4.7 / 2.89 |  |  |  |
|  | TOTAL | 849.1M³ 818.0MT |  | TOTAL | 359.8 M³ 314.0MT |

Remarks (Incl. Consumption per 24 Running Hrs for Main Engine / Aux Engines / Main & Aux Boilers)

Main engine HFO = 26.09mt; Main engine MGO = 4.20mt; Auxiliary engine HFO =1.92mt; Auxiliary engine

MGO = 2.09 mt; Auxiliary boiler HFO = 0.9mt; Auxiliary boiler MGO = 2.98mt; Thermal oil boiler HFO = 6.87mt;

Thermal oil boiler MGO = 2.41 mt

P.Maiti			                                  Gelu Antin

| Name Chief Engineer |  | Name Master |
| --- | --- | --- |