Vessel:  Bitu Atlantic		    Date (dd/mm/yy):   31/12/18		Name : 2nd Engineer : ASHOK JOLLY

| Tag No | Equipment ( Shackle / Strap / Wire Sling / Chain Block etc) | S.W.L (Tons) | Condition Upon Visual Inspection (top hook, lifting hook, Swivel, Heaving chain, cover block, chain w/hook etc) | Last Load Test Done (Cert to be kept in Chain register) | Suitability for Use (Fit / Unfit) | Remarks |
| --- | --- | --- | --- | --- | --- | --- |
| Wire Sling - 1 | 3 pcs | 0.8 tons | Size = 10 mm, Length = 1.5 mtrs, New | 11416CHN15-1 | Fit for use |  |
| Wire Sling – 2 | 2 pcs | 1.0 tons | Size = 10 mm, Length = 0.4 mtrs, New |  | Fit for use |  |
| Wire Sling – 3 | 2 pcs | 1.0 tons | Size = 12 mm, Length = 1.5 mtrs, New |  | Fit for use |  |
| Wire Sling – 4 | 2 pcs | 2.0 tons | Size = 16 mm, Length = 1.0 mtrs, New |  | Fit for use |  |
| Wire Sling – 5 | 2 pcs | 2.0 tons | Size = 16 mm, Length = 3.0 mtrs, New |  | Fit for use |  |
| Nylon Strop-1 | 1 pc | 2.0 tons | Belt type, width = 50 mm, length = 3.0 mtrs, New |  | Fit for use |  |
| Nylon Strop-2 | 1 pc | 3.2 tons | Belt type, width = 50 mm, length = 1.5 mtrs, New |  | Fit for use |  |
| Nylon Strop-3 | 1 pc | 2.0 tons | Belt type, width = 50 mm, length = 3.0 mtrs, New |  | Fit for use |  |
| Chain Block-1 | 2 pcs | 0.75 tons | D lever type, New |  | Fit for use |  |
| Chain Block-2 | 2 pcs | 1.0 tons | New |  | Fit for use |  |
| Chain Block-3 | 1 pc | 2.0 tons | New |  | Fit for use |  |
| Shackle 1 | 2 pcs | 2.0 tons | D shackle, New |  | Fit for use |  |
| Tag No | Equipment ( Shackle / Strap / Wire Sling / Chain Block etc) | S.W.L (Tons) | Condition Upon Visual Inspection (top hook, lifting hook, Swivel, Heaving chain, cover block, chain w/hook etc) | Last Load Test Done (Cert to be kept in Chain register) | Suitability for Use (Fit / Unfit) | Remarks |
| Shackle 2 | 1 pc | 2.5 tons | D shackle, New |  | Fit for use |  |
| Shackle 3 | 5 pcs | 3.0 tons | D shackle, New |  | Fit for use |  |
| Shackle 4 | 1 pc | 1.5 tons | D shackle, New |  | Fit for use |  |